Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Auto Shop Automotive Lift Operations.

Hazards: Working with lubricants, flammable liquids, potential ignition sources, vapors, and other potential inhalation hazards inherent to the Auto Working processes. Hazards include pinch, crush, puncture, struck by, and strain/sprain.

WARNING: DO NOT use this machine unless you have been trained in its safe use and operation. Failure to comply could result in serious injury.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE): eye protection, including protective glasses or goggles, hearing protection (when equipment is in use) and long pants.  Hearing protection can include earmuffs or ear plugs based on the sound level.

- Mechanical gloves shall be worn as required.

- Tie back hair, no loose clothing and no jewelry should be worn.

Before AUTO SHOP- AUTOMOTIVE LIFT OPERATIONS

  - Ensure workspaces, table, and walkways are free and clear of clutter and debris.

  - Ensure all warning labels are accessible and readable.

  - Confirm that the lifts rated load capacity is visible, and the vehicle is within the rated capacity.

  - Check that all lift controls are operating properly.

  - Make sure vehicle in park or chocks restrict rolling.

  - Check for damaged or excessive wear on all parts and equipment.

  - Look for evidence of hydraulic or pneumatic leaks (where applicable).

  - Ensure all monthly and annual inspections and reviews are posted and up to date.

During AUTO SHOP- AUTOMOTIVE LIFT OPERATIONS

  - Check overhead clearance before lifting.

  - Close the vehicle doors. Make sure no one is inside before raising the vehicle.

  - Position the vehicle so the center of gravity is balanced on the lift according to manufacturer’s recommendations.

  - One person is responsible for working the controls and verifying the stability and clearances of the vehicle.

  - Keep people clear of vehicle when lifting and lowering.

  - Do not leave the controls unattended while the lift is in motion.

  - Do not work on the vehicle while it is being raised.

  - Minimize the movement of the vehicle on the lift while working on it.

  - Use only designated, standard blocks, support equipment or stands when required

After AUTO SHOP- AUTOMOTIVE LIFT OPERATIONS have been completed

  - Collect all waste/debris and place in appropriate containers

  - Inspect equipment to ensure no damage has occurred, and accessories.

  - If equipment is damaged or inoperable follow lockout procedures.