Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Auto Shop Parts Washer Operations.

Hazards: Working with aqueous solutions, lubricants, flammable liquids, potential ignition sources, vapors, and other potential inhalation hazards inherent to the Auto Working processes.

WARNING: DO NOT use this machine unless you have been trained in its safe use and operation. Failure to comply could result in severe injury.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE): eye protection, including protective glasses or goggles, hearing protection (when equipment is in use) and long pants.  Hearing protection can include earmuffs or ear plugs based on the sound level.

- Chemical resistant gloves shall be worn when cleaning parts.

- Tie back a hair, no loose clothing and no jewelry should be worn.

Before AUTO SHOP PARTS WASHER OPERATIONS

  - Ensure workspaces, table, and walkways are free and clear of clutter and debris.

  - Use only in a well-ventilated area.

  - Avoid contact with cleaning fluid and vapor.

  - Do not modify equipment.

  - Do not replace, modify or obstruct the fusible link

  - Do not obstruct or hang anything on the lid.

During AUTO SHOP PARTS WASHER OPERATIONS

  - Do not place hot parts in contact with cleaning fluid.

  - Do not operate machine in a manner that will cause splashing or cause mist to form.

  - Do not operate equipment in proximity to potential ignition sources.

  - Do not operate parts washers on wet surfaces or expose to rain.

  - Keep hands away from tank top edge, do not lean into equipment.

  - Do not operate equipment if a leak is apparent or occurs during use.

  - Do not leave machine running while unattended.

After AUTO SHOP PARTS WASHER OPERATIONS have been completed

  - Collect all waste/debris and place in appropriate containers.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable follow lockout procedures.