Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Auto Shop Tire Changer Operations.

Hazards: Working with heavy materials, lubricants, flammable liquids, potential ignition sources, vapors, and other potential inhalation hazards inherent to the Auto Working processes. Hazards include pinch, crush, puncture, strain/sprain, inhalation, and struck by flying debris or objects.

WARNING: DO NOT use this machine unless you have been trained in its safe use and operation. Failure to comply could result in severe injury.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE): eye protection, including protective glasses or goggles, hearing protection (when equipment is in use) and long pants.  Hearing protection can include earmuffs or ear plugs based on the sound level.

- Tie back hair, no loose clothing and no jewelry should be worn.

Before AUTO SHOP TIRE CHANGER OPERATIONS

  - Ensure workspaces, and walkways are free and clear of clutter and debris.  Tires shall not be inflated beyond the manufacturer’s recommendation or a maximum of 40 PSI.

  - Tires shall be completely deflated before adjusting the tire, rim or wheel being serviced.

  - Remove and dispose of any bent, corroded, broke or cracked rims, wheel or rim or wheel components.

  - Always replace damaged or leaky valves.

  - Always use a safety cage when inflating tires with locking rings on the rim.

  - Make sure guards are present, installed and working.

  - Tool rests should be adjusted to within 1/8 in. of the wheels

  - Locate and be aware of the operation of the ON/OFF switch or E-stop.

  - Disconnect power supply before changing or adjusting tools and equipment.

  - Follow correct clamping procedures to ensure work is secure.

During AUTO SHOP TIRE CHANGER OPERATIONS

  - Never stand within the trajectory of the tire, rim or wheel components during the inflation process.

  - Always use the proper tools for the type of rim or wheel being serviced.

  - Never apply heat to a wheel or wheel component that I mounted to an inflated tire.

  - Only mount tires on compatible wheels of matching bead diameter and width.

  - Keep all covers and guards on the machine when it is on and in operation.

  - Do not leave machine running while unattended.

  - Shut OFF the power supply to the motor before mounting or removing accessories.

After AUTO SHOP TIRE CHANGER OPERATIONS have been completed

  - Collect all waste/debris and place in appropriate containers.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable follow lockout procedures.