Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Baking Metal Operations.

Hazards: Work involving Baking Metal Operations produce high temperatures and heat and can create fires, and cause burns to workers.  In addition, inhalation hazards due to exposures to flammable and toxic vapors inherent in the Metal Baking processes.

WARNING: DO NOT use this machine unless you have been trained in its safe use and operation. Failure to comply could result in severe injury and even death.

General Safety

- Never start work without proper authorization and training.

- Personal Protective Equipment (PPE): leather or fire-retardant gloves, eye & face protection, including protective helmet with protective lenses rated at a minimum of #6 shade, long pants (leather or fire-retardant), leather apron.  Any other PPE required as per current SDS of any hazardous product(s) being used.

- Tie back and cover hair, no loose clothing and no jewelry should be worn.

Before METAL BAKING OPERATIONS

  - Ensure all combustible and flammable liquids and materials have been moved away or covered.

  - Always have fire-fighting equipment (fire extinguisher) on hand before starting.

  - Inspect equipment, ensure they are protected, labeled, and secure.

  - Inspect any and all heat producing devices, check refractory if applicable, interior and exterior of device for any damage.

  - Inspect Electrical cables, gas connections, shut offs and connections.

  - Control access in and around work areas

  - Ensure ventilation is turned on and operational.

During FOUNDRY OPERATIONS

  - Keep people away from operation who are not wearing PPE.

- Ensure heat-producing devices and other devices are always kept clear.

- Take particular care to avoid metal coming into contact with other materials.

- Do not leave machine running while unattended.

  - Once work is completed shut down equipment including Metal Baking Device and turn off power supply.

After METAL BAKING OPERATIONS has been Completed

  - Ensure walkways and workspaces are clear and clean of any accumulated materials, slag, etc.

  - Collect all combustibles/waste/debris and place in appropriate containers.

  - Allow the equipment to cool down before handling.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable, follow lockout procedures.