Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Forge Operations.

Hazards: Work involving Forges & Furnaces produce high temperatures and heat & can create fires, and cause burns to workers.  In addition, inhalation hazards due to exposures to flammable & toxic vapors inherent in the Forging or Heating processes.

WARNING: DO NOT use this machine unless you have been trained in its safe use and operation. Failure to comply could result in severe injury and even death.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE): leather or fire-retardant gloves, eye & face protection, including protective helmet with protective lenses rated at a minimum of #6 shade, long pants (leather or fire-retardant), leather apron.  Any other PPE required as per current SDS of any hazardous product(s) being used.

- Tie back and cover hair, no loose clothing and no jewelry should be worn.

Before FORGE OPERATIONS

  - Ensure all combustible and flammable liquids and materials have been moved away or covered.

  - Always have fire-fighting equipment (fire extinguisher) on hand before starting.

  - Inspect equipment, ensure they are protected, labeled and secure.

  - Inspect the Forge, or Furnace device check refractory, interior and exterior of device for any damage.

  - Inspect Electrical cables, gas connections, shut offs and connections.

  - Ensure standard grounding is provided.

  - Ensure screens are in place around work areas

  - Ensure ventilation is provided and operational.

During FORGE OPERATIONS

  - Keep people away from operation who are not wearing PPE.

  - Protect all connections and equipment.

  - Ensure areas of operation moving of equipment, pots and other devices are always kept clear.

  - Take particular care to avoid metal splash and contact with other materials.

- Do not leave machine running while unattended.

  - Once work is completed shut down equipment, forge & disconnect power supply.

After FORGE OPERATIONS has been Completed

  - Ensure walkways and workspaces are clear and clean of any accumulated materials, slag, etc.

  - Collect all combustibles/waste/debris and place in appropriate containers.

  - Allow the equipment to cool down before handling.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable follow lockout procedures.