Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Foundry Operations.

Hazards: Work involving Foundries produce high temperatures and heat & can create fires, and cause burns to workers.  In addition, inhalation hazards due to exposures to flammable & toxic vapors inherent in the Foundry processes.

WARNING: DO NOT use this machine unless you have been trained in its safe use and operation. Failure to comply could result in severe injury and even death.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE): leather or fire-retardant gloves, eye & face protection, including protective helmet with protective lenses rated at a minimum of #6 shade, long pants (leather or fire-retardant), leather apron.  Any other PPE required as per current SDS of any hazardous product(s) being used.

- Tie back and cover hair, no loose clothing and no jewelry should be worn.

Before FOUNDRY OPERATIONS

  - Ensure all combustible and flammable liquids and materials have been moved away or covered.

  - Always have fire-fighting equipment (fire extinguisher) on hand before starting.

  - Inspect equipment, ensure they are protected, labeled and secure.

  - Inspect the Foundry and related devices, check refractory if applicable, interior and exterior of device for any damage.

  - Inspect Electrical cables, gas connections, shut offs and connections.

  - Inspect crucibles.

  - Control access in and around work areas

  - Have dry sand & shovel ready; NEVER use water on metal fires (causes explosions.

  - Ensure ventilation is provided and operational.

During FOUNDRY OPERATIONS

  - Keep people away from operation who are not wearing PPE.

- Never pour over wet ground; concrete must be sand-covered; use dry sand to control spills.

- Ensure heat producing devices and other devices are always kept clear.

  - Take particular care to avoid metal splash and contact with other materials.

- Do not leave machine running while unattended.

  - Once work is completed shut down equipment, foundry and turn off power supply.

After FORGE OPERATIONS has been Completed

  - Ensure walkways and workspaces are clear and clean of any accumulated materials, slag, etc.

  - Collect all combustibles/waste/debris and place in appropriate containers.

  - Allow the equipment to cool down before handling.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable follow lockout procedures.