Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Media Blaster Operations.

Hazards: Work involving fine metal filings, airborne dust and potential inhalation hazards inherent to Metal Working processes. Hazards include pinch, crush, inhalation, and struck by flying debris or objects.

NOTE: Media Blaster Units are typically fully enclosed and contained.

WARNING: DO NOT use this device unless you have been trained in its safe use and operation. Failure to comply could result in severe injury.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE): eye protection, including face shield and protective glasses or goggles, hearing protection (when equipment is in use) and long pants.  Hearing protection can include earmuffs or ear plugs based on the sound level.

- Tie back and cover hair, no loose clothing and no jewelry should be worn.

Before MEDIA BLASTER OPERATIONS

  - Ensure workspaces, table, and walkways are free and clear of clutter and debris.

  - All blast cabinets must be grounded to avoid shock.

  - Electrical cords must be three (3) prongs grounded.

  - Ensure access devices, gloves and interior light fixtures are in good condition.

  - Ensure attached filter system is attached and in good condition.

  - Inspect fittings and hoses for damage and wear.

  - Check the gaskets on all doors.

  - Locate and be aware of the operation of the ON/OFF switch or E-stop.

  - Disconnect power supply before changing or adjusting tools and equipment.

During MEDIA BLASTER OPERATIONS

  - Ensure that work is secure.

  - Keep all covers and access doors on the machine when it is on and in operation.

  - Only operate the unit with all doors securely closed and with the dust collector system turned on.

  - If you need to stop the media blaster to make adjustments, do not open access doors till unit has come to a complete stop.

  - Do not leave machine running while unattended.

  - Shut OFF the power supply to the motor to stop before mounting/removing accessories.

After MEDIA BLASTER OPERATIONS have been completed

  - Collect all waste/debris and place in appropriate containers.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable follow lockout procedures.