Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Metal Chop Saw Operations.

Hazards: Work involving fine metal filings, airborne dust and potential inhalation hazards inherent to Metal Working processes. Hazards include pinch, crush, and struck by flying debris or objects.

WARNING: DO NOT use this machine unless you have been trained in its safe use and operation. Failure to comply could result in severe injury.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE): eye protection, including protective glasses or goggles, hearing protection (when equipment is in use) and long pants.  Hearing protection can include earmuffs or ear plugs based on the sound level.

- Tie back hair, no loose clothing and no jewelry should be worn.

- Do Not wear gloves when operating a chop saw.

Before METAL CHOP SAW OPERATIONS

  - Make sure all blades are sharp, balanced, tracking correctly and fastened securely.

  - Ensure no slip/trip hazards are present in workspaces and walkways.

  - Make sure guards are present, installed and working.

  - Locate and be aware of the operation of the ON/OFF switch or E-stop.

  - Disconnect power supply before changing or adjusting tools and equipment.

  - Make sure the guards operate as required.

  - Remove all adjusting keys, wrenches and tools used in the setup from the work area.

  - Use hold down devices when required.

During METAL CHOP SAW OPERATIONS

  - Ensure that work is secure.

  - Keep one hand on the trigger switch and handle and use the other hand to hold the stock against the fence.

  - Keep hands out of the path of the blade.

  - Allow motor to reach full speed before cutting.

  - If you need to stop cutting, hold the material firmly and shut down the saw. Wait for the blade to stop moving before removing the piece.

  - Only cut one work piece at a time.

  - Shut OFF the power supply to the motor before mounting or removing accessories.

  - Do not leave machine running while unattended.

  - Shut OFF the power supply to the motor before mounting/removing accessories or removing scrap pieces from work area.

After METAL CHOP SAW OPERATIONS have been completed

  - Collect all waste/debris and place in appropriate containers.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable follow lockout procedures.