Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Metal Milling Machine Operations.

Hazards: Work involving fine metal filings, airborne dust and potential inhalation hazards inherent to Metal Working processes.  Hazards include pinch, crush, inhalation, and struck by flying debris or objects.

WARNING: DO NOT use this machine unless you have been trained in its safe use and operation. Failure to comply could result in severe injury.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE): eye protection, including protective glasses or goggles, hearing protection (when equipment is in use) and long pants.  Hearing protection can include earmuffs or ear plugs based on the sound level.

- Tie back hair, no loose clothing and no jewelry should be worn.

- Wear gloves when handling milling cutters only.

- When in operation do not wear gloves in or around the revolving milling cutter.

Before METAL MILLING MACHINE OPERATIONS

  - Ensure workspaces, table, and walkways are free and clear of clutter and debris.

  - Make sure guards are present, installed and working.

  - Ensure work is properly secured in place.

  - Make sure bolts that are used to hold down work clear the tools in operation.

  - Ensure table stops, tooling and supported pieces are properly tightened in position.

  - Locate and be aware of the operation of the ON/OFF switch or E-stop.

  - Disconnect power supply before changing or adjusting tools and equipment.

  - Make sure the guards operate as required.

  - Remove all adjusting keys, wrenches and tools used in the setup from the work area.

During METAL MILLING MACHINE OPERATIONS

  - Ensure that work is secure.

  - Keep all covers and guards on the machine when it is on and in operation.

  - Keep hands away from all rotating cutters.

  - Keep all hands, fingers and body parts away from the revolving milling cutter.

  - Use the correct cutting fluids.

  - Allow motor to reach full speed before starting the milling processes.

  - If you need to stop the milling machine, shut down the machine using the brake after power has been turned off.

  - Do not leave machine running while unattended.

  - Shut OFF the power supply to the motor and allow rotation to stop before mounting/removing accessories.

After METAL MILLING MACHINE OPERATIONS have been completed

  - Collect all waste/debris and place in appropriate containers.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable follow lockout procedures.