Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Planer Operations.

Hazards: Work involving fine wood dust, wood chips and other combustible materials. Hazards include pinch, crush, inhalation hazard, and struck by flying debris or objects.

WARNING: DO NOT use this machine unless you have been trained in its safe use and operation. Failure to comply could result in severe injury.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE): eye protection, including protective glasses or goggles, hearing protection (when equipment is in use) and long pants.  Hearing protection can include earmuffs or ear plugs based on the sound level.

- Tie back and cover hair, no loose clothing and no jewelry should be worn.

- Do Not wear gloves when operating a planer.

Before PLANER OPERATIONS

  - Are all blades, knives adjusted with the proper clearance.

  - Are all blades sharp, balanced and fastened securely.

  - Ensure workspaces, table, and walkways are free and clear of clutter and debris.

  - Make sure guards are present, installed and working.

  - Locate and be aware of the operation of the ON/OFF switch or E-stop.

  - Disconnect power supply before changing or adjusting tools and equipment.

  - Make sure the guards operate as required and adjust to the stock as it is being run through the planer.

  - Remove all wrenches and tools used in the setup from the work area.

  - Use hold down or push blocks.

During PLANER OPERATIONS

  - Ensure that work is secure.

  - Use hold-down push blocks for beveling and surface operations.

  - Maintain an adequate amount of downward and forward force with push blocks as the knife blades on a revolving cutting head can take the stock from an operator’s hands

  - Do not leave machine running while unattended.

  - Shut OFF the power supply to the motor and allow rotation to stop before mounting/removing accessories.

After PLANER OPERATIONS have been completed

  - Collect all waste/debris and place in appropriate containers.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable follow lockout procedures.