Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Post Sander Operations.

Hazards: Work involving fine wood dust, wood chips and other combustible materials. Hazards include pinch, crush, inhalation hazard, and struck by flying debris or objects.

WARNING: DO NOT use this machine unless you have been trained in its safe use and operation. Failure to comply could result in sever injury.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE): eye protection, including protective glasses or goggles, hearing protection (when equipment is in use) and long pants.  Hearing protection can include earmuffs or ear plugs based on the sound level.

- Tie back and cover hair, no loose clothing and no jewelry should be worn.

- Do Not wear gloves when operating a sander.

Before POST SANDING OPERATIONS

  - Ensure workspaces, table, and walkways are free and clear of clutter and debris.

  - Check the integrity of the sleeves. Any ripped sleeves mut be replaced.

  - Make sure gap between the sanders table and the moving disk is kept as small as possible.

  - Locate and be aware of the operation of the ON/OFF switch or E-stop.

  - Disconnect power supply before changing or adjusting tools and equipment.

  - When adjusting the post sander, be sure the sander is supported and positioned to avoid contact with the operator.

  - Adjust all work rests on all manually fed sanders to provide minimum clearance between the belt and the rest device.

During POST SANDING OPERATIONS

  - Ensure that work is mounted securely.

  - Distances between fingertips and the moving disk should be no closer than 8cm.

  - Do not sand pieces that are to small to be supported.

  - Always hold the work firmly when sanding.

  - Do not leave machine running while unattended.

  - Shut OFF the power supply to the motor and allow rotation to stop before mounting/removing accessories.

After POST SANDING OPERATIONS have been completed

  - Collect all waste/debris and place in appropriate containers.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable follow lockout procedures.