Purpose: To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Powder Coating Operations.

Hazards: Work involving Powder Coating Operations can produce large amounts of residual dust and over spray fires, compressed air exposures and housekeeping concerns.  In addition, inhalation hazards due to exposures to toxic vapors and airborne particulate in the Powder Coating processes.

WARNING: DO NOT operate powder coating equipment unless you have been trained in its safe use and operation. Failure to comply could result in severe injury and even death.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE):  chemical resistant gloves, long sleeves, pants and closed-toe shoes and safety goggles.   When working with powder and when doors to unit are open staff must wear a respirator with P100 cartridges or equivalent.   Also, any other PPE required as per current SDS of any hazardous product(s) being used.

- Tie back and cover hair, no loose clothing and no jewelry should be worn.

Before POWDER COATING OPERATIONS

  - Ensure all combustible and flammable liquids and materials have been moved away or covered.

  - Always have fire-fighting equipment (fire extinguisher) on hand before starting.

  - Inspect equipment, ensure they are protected, labeled and secure.

  - Ensure cabinet doors are shut before power coater is operated.

  - Inspect Electrical connections, compressors, shut offs and connections.

  - Ensure all parts, racks, and equipment are properly grounded to prevent static buildup.

  - Use designated ovens specifically for completing the powder coating process.

  - Ensure ventilation is turned on and operational.

During POWDER COATING OPERATIONS

  - Keep people away from operation who are not wearing PPE.

- Ensure all hoses, hoppers, spray guns are always kept clear.

- Stand outside booths while spraying.

- Never spray towards another operator.

- Do not leave powder coating equipment running while unattended.

  - Once work is completed shut down equipment before opening the powder coater cabinet. Turn off power supply.

After POWDER COATING OPERATIONS has been Completed

  - Collect/clean all combustibles/waste/debris (overspray) and place in appropriate containers.  Use an industrial vacuum with HEPA filtration for cleanup. Do not dry sweep.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable follow lockout procedures.