Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Scroll Saw Operations.

Hazards: Work involving fine wood dust, wood chips and other combustible materials. Hazards include cut, pinch, crush, inhalation, and struck by flying debris or objects.

WARNING: DO NOT use this machine unless you have been trained in its safe use and operation. Failure to comply could result in severe injury.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE): eye protection, including protective glasses or goggles, hearing protection (when equipment is in use) and long pants.  Hearing protection can include earmuffs or ear plugs based on the sound level.

- Tie back and cover hair, no loose clothing and no jewelry should be worn.

- Do Not wear gloves when operating a scroll saw.

Before SCROLL OPERATIONS

  - Ensure workspaces, table, and walkways are free and clear of clutter and debris.

  - Make sure blade is positioned properly and the table-top is smooth and polished.

  - Select the correct speed and type of blade for the work you are doing.

  - Locate and be aware of the operation of the ON/OFF switch or E-stop.

  - Disconnect power supply before changing or adjusting tools and equipment.

  - Remove all wrenches and tools used in the setup from the work area.

During SCROLL SAW OPERATIONS

  - Ensure that work and equipment is secure.

  - Hold the stock firmly and flat on the table to prevent stock from turning and drawing your fingers into the blade.

  - Keep your hands, fingers and body parts well out of the way of the blade.

  - If you need to stop cutting, hold the material firmly and shut down the saw. Wait for the blade to stop moving before removing the piece.

  - Do not leave machine running while unattended.

  - Shut OFF the power supply to the saw and allow to stop before mounting/removing accessories.

After SCROLL SAW OPERATIONS have been completed

  - Collect all waste/debris and place in appropriate containers.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable follow lockout procedures.