Purpose:  To provide SD 62 staff, with a list of procedures to ensure they are protected against potential hazards associated with Wood Lathes.

Hazards: Work involving fine wood dust, wood chips and other combustible materials. Hazards include pinch, crush, inhalation, entanglement and struck by flying debris or objects.

WARNING: DO NOT use this machine unless you have been trained in its safe use and operation. Failure to comply could result in severe injury.

General Safety

- Never start work without proper authorization & training.

- Personal Protective Equipment (PPE): eye protection, including face shield and protective glasses or goggles, hearing protection (when equipment is in use) and long pants.  Hearing protection can include earmuffs or ear plugs based on the sound level.

- Tie back and cover hair, no loose clothing and no jewelry should be worn.

- Do Not wear gloves when operating a drill press.

Before WOOD LATHE OPERATIONS

  - Ensure workspaces, table, and walkways are free and clear of clutter and debris.

  - Make sure guards are present, installed and working.

  - Locate and be aware of the operation of the ON/OFF switch or E-stop.

  - Disconnect power supply before changing or adjusting cutting tools.

  - Check that all materials is tightly clamped in the chuck or drive device.

  - Ensure correct speed for machining process is selected.

  - Remove any chuck tightening devices before starting the lathe.

During WOOD LATHE OPERATIONS

  - Ensure that work is mounted securely in the chuck.

  - During milling & cutting operations: position the cutting tool to correct angle and height for operation.

  - Follow job specifications for the speed, feed and depth of cut for being turned.

  - Place all tools, measuring instruments and other objects on a bench before starting the lathe.

  - Ensure that all chip shields are in place (if equipped).

  - Stop lathe before taking measurements or making adjustments of any kind.

  - Do not leave machine running while unattended.

  - Shut OFF the power supply to the motor and allow rotation to stop before mounting/removing accessories.

After WOOD LATHE OPERATIONS have been completed

  - Collect all waste/debris and place in appropriate containers.

  - Inspect equipment to ensure no damage has occurred, guards and other devices are in good condition and in place. Record any damage as per the Technology Education Safety Manual.

  - If equipment is damaged or inoperable follow lockout procedures.